SITE-SPECIFIC FIRE RISK ASSESSMENT

| Site: | Ivy House, 23 Friarn Street, Bridgwater TA6 3LH |
| --- | --- |
| Client: | GG-296-739 Limited |
| Principal Contractor: | Oxygen Management Ltd |
| Assessment Date: | 1st February 2026 |
| Assessor: | Site Manager |
| Review Date: | Monthly or after any significant change |

# Step 1: Identify Sources of Ignition

- Hot works (soldering, welding, cutting) - Permit to Work system in place

- Electrical equipment and temporary electrical installations

- Smoking (prohibited inside all buildings and within 5m of entrances)

- Space heaters (only approved types with thermostatic cut-off permitted)

- LPG cylinders and flammable gas supplies

- Portable generators and compressors

- Arson - site secured and locked when unattended

# Step 2: Identify Sources of Fuel

- Timber, MDF and other combustible building materials

- Packaging materials and waste (cardboard, polythene wrapping)

- Paints, solvents, adhesives and other flammable liquids (COSHH store)

- LPG cylinders for plumbing and heating works

- Insulation materials (PIR, mineral wool packaging)

- Dust from cutting and sanding operations

- Waste stored in skips

# Step 3: People at Risk

- All site operatives and sub-contractors

- Site visitors (clients, architects, building control)

- Delivery drivers

- Members of the public passing the site on Friarn Street

- Occupants of neighbouring properties

# Step 4: Control Measures

## 4.1 Means of Escape

- Primary escape route identified and communicated at site induction

- Secondary escape route available and clearly signed

- All escape routes kept clear of materials, plant and obstructions at all times

- Emergency exit signage installed and illuminated where necessary

- Assembly point established at: [to be confirmed on site - street frontage area]

## 4.2 Fire Detection and Warning

- Manual call points installed at key locations

- Air horn/klaxon available as temporary fire alarm during construction phase

- All operatives made aware of alarm sound during site induction

- Existing building fire alarm system: [confirm status - isolated/active during works]

## 4.3 Fire Fighting Equipment

- Fire extinguishers provided at strategic locations throughout the site:

- - Minimum 2 x 6kg dry powder at each floor level

- - 1 x CO2 extinguisher at temporary electrical distribution board

- - Task-specific extinguishers provided by sub-contractors for hot works

- All extinguishers serviced annually and inspected monthly

- Extinguisher locations marked with appropriate signage

- All site personnel trained in basic extinguisher use during induction

## 4.4 Storage and Housekeeping

- Flammable liquids stored in dedicated, ventilated COSHH storage container

- LPG cylinders stored externally in a secure, ventilated cage

- Waste cleared daily - no combustible waste left inside the building overnight

- Skip located away from the building (minimum 6m where possible)

- Good housekeeping enforced - daily site clean-up before leaving

## 4.5 Emergency Procedures

- On discovering a fire: raise the alarm, evacuate to assembly point, call 999

- Do not attempt to fight the fire unless trained and it is safe to do so

- Site Manager to conduct roll call at assembly point using sign-in sheet

- Emergency services: Fire - 999 | Ambulance - 999 | Police - 999

- Nearest A&E: Musgrove Park Hospital, Taunton (approx. 12 miles)

- Fire management responsibilities communicated at every site induction

- Fire drills to be conducted periodically throughout the project

# Step 5: Review

This assessment will be reviewed monthly, after any fire incident or near miss, and whenever there is a significant change to the works or site layout.

| Review Date | Reviewed By | Changes Made | Signature |
| --- | --- | --- | --- |